# Closing the muon-catalysed fusion energy gap: direct levers and a subcritical blanket

[Author names to be added]

Technical note, October 2026

Abstract

Muon-catalysed d–t fusion (μCF) is the only established route to fusion at room temperature. With ~150 fusions per muon and ~5 GeV per muon, it reaches a scientific gain of ~0.53–0.67 and an electric gain of ~0.11, i.e. ~9× short of net electricity. We quantify two ways to close that gap. Directly: cheaper muons (/2.5), halved sticking, a 3× faster cycle and better conversion together reach an electric gain of ~1.1. Indirectly: using the 14.1 MeV fusion neutrons to drive a subcritical fission blanket gives electric gains of several units even with today’s μCF numbers. But today μCF yields ~30 neutrons per GeV of beam, the same as proton spallation (20–35 n/GeV), which accelerator-driven systems already use. A μCF hybrid is therefore worth building only when muons cost ≲4–8 GeV or catalyse ≳100–175 fusions each. With all direct levers it would reach ~170 n/GeV, 5–8× spallation. With sourced muon-cost scenarios (2–3 GeV) and the fusions-per-muon values of recent polarisation studies, net electricity is reached only at the threshold (gain ≈ 1.0–1.1), and only with 50%/60% conversion efficiencies. Either way, muon production cost and alpha-sticking are the deciding quantities, and high-density measurements now under way (MuFusE, PSI) bear directly on the second.

# 1. Starting point

A muon in a d–t mixture catalyses fusions at the cycling rate φλc until it decays or sticks to an alpha particle (probability ωs per fusion), so the number of fusions per muon is X = 1/(λ₀/φλc + ωs). Calibrated to the measured X ≈ 150 at ωs = 0.45%, the cycle model reproduces the published gains: Q ≈ 0.53 from fusion alone, 0.67 with lithium-blanket breeding heat, and an electric gain of ≈ 0.11 at 40% thermal and 40% accelerator efficiency (published: ~14%).

Electric gain of pure μCF vs effective sticking and muon cost (40% thermal and accelerator efficiency, lithium blanket).

# 2. Direct levers

| Scenario | Fusions per muon | Scientific Q | Electric gain |
| --- | --- | --- | --- |
| Today (5 GeV/μ, ω_s = 0.45%) | 150 | 0.67 | 0.11 |
| Muon cost /2.5 (2 GeV/μ) | 150 | 1.68 | 0.27 |
| Sticking halved | 226 | 1.01 | 0.16 |
| Cycling ×3 | 191 | 0.86 | 0.14 |
| Cost /2.5 + sticking /2 + cycling ×3 | 336 | 3.77 | 0.60 |
| … + better conversion (η_th 0.5, η_acc 0.6) | 336 | 3.77 | 1.13 |

No single lever is enough. Net electricity needs all four at once.

# 3. Subcritical blanket

Each fusion emits a 14.1 MeV neutron. In a blanket with keff<1, a source neutron triggers k/(ν(1-k)) fissions (3.6 at k=0.90, 7.6 at 0.95, 19.6 at 0.98), each releasing ~200 MeV. At keff=0.95, a typical accelerator-driven-system design value, the electric gain becomes:

| Neutron source | Neutrons per GeV of beam | Electric gain at k_eff = 0.95 |
| --- | --- | --- |
| Proton spallation on lead (ADS) | 20–35 | ~4–9 |
| μCF today | 30 | 7.4 |
| μCF, muon cost /2.5 | 75 | 18.5 |
| μCF, all direct levers | 168 | 41.3 |

Electric gain of a subcritical blanket driven by μCF or by spallation, vs k_eff.

Absolute gains are optimistic (the multiplication formula ignores (n,2n), parasitic capture and leakage), but the comparison between sources is robust because the same blanket multiplies every source’s neutrons equally. At keff=0.95 the fusion itself supplies only ~1.1% of the heat. Tritium is not a constraint: each source neutron produces ~12 spare neutrons beyond those absorbed in fission, while breeding needs ~1 per fusion.

# 3.1 How cheap must muons get? (notebook 16)

We turn the question around and ask what beam energy per stopped muon, Eμ, each goal allows. With today’s cycle (X ≈ 150), net electricity needs Eμ ≈ 0.54–0.62 GeV, 8–9× cheaper than today’s ~5 GeV. That would take 1.6 stopped muons per GeV of beam, 23% of the ceiling set by the π⁻ rest mass. Cheaper muons alone are not a realistic route.

Using sourced scenarios: muon cost 5 GeV today, 2 GeV for Kelly et al. (2021) and 3.0 GeV for a simulated active-target source. Fusions per muon follow Yin et al. (2026): 148 unpolarised, 193 polarised and 300–350 for their “multi-technology synergy” case. At today’s conversion efficiencies (0.4/0.4), no scenario reaches net electricity; the best supported case gives 0.58. With 0.5/0.6, a 2 GeV source at X ≈ 325 reaches gain 1.09, and a 3.0 GeV active target with beam-heat recovery reaches 0.97. Both are at the threshold. Clear margin needs the extrapolated engineering-limit cycle (X ≈ 550; gain 1.85 at 2 GeV).

Recovering beam heat from the pion-production target raises a 5 GeV source from 0.11 to 0.23. This is one ingredient of Kelly et al.’s 2.5× reduction, not an extra lever on top of it.

As a neutron source, μCF ties spallation today (break-even 5.4 GeV per muon). With polarised fuel it wins below 7.0 GeV per muon, so for waste burning the cycle measurements matter more than muon cost.

# 4. Conclusions

- Pure μCF is ~9× from net electricity. Closing it directly needs cheaper muons, lower sticking, faster cycling and better conversion together.

- A subcritical blanket gives net electricity with today’s μCF numbers, but so does spallation, at the same neutrons per GeV. μCF earns its place only once it out-produces spallation: muons below ~4–8 GeV, or ≳100–175 fusions per muon.

- The two numbers to watch are muon production cost (accelerator, pion-production target and capture design) and effective sticking at high density (MuFusE).

- A μCF hybrid is a neutron-source technology. It carries the waste, licensing and safety profile of any accelerator-driven fission system.

Reproducibility. Notebooks 09, 10 and 16 at https://github.com/m1110/cold-fusion-gap. Inputs and their verification status: VERIFICATION.md (μCF data and muon cost at secondary/abstract level; spallation yield from GEANT4 studies, abstract-level).

Acknowledgements. Analysis and drafting assisted by an AI model (Claude, Anthropic).

# References (to be checked against primary sources)

- Jones S. E. et al., Phys. Rev. Lett. 56, 588 (1986): LAMPF μCF measurements.

- Kelly K. et al., J. Phys. Energy 3, 035003 (2021): muon production cost and μCF energy balance.

- Yin X., Kou W., Chen X., Muon-catalyzed nuclear fusion: physical mechanism, bottleneck breakthroughs and an engineering pathway, arXiv:2605.26432 (2026) (read in full).

- Parisi J. F., Rutkowski A., Isotope production in muon-catalyzed-fusion systems, arXiv:2511.20951 (2025) (read in full).

- Jändel M., Design principles for efficient muon production in muon-catalyzed fusion reactors, J. Fusion Energy 8, 81 (1989).

- MuFusE collaboration, arXiv:2606.05333 (2026): diamond-anvil cell for μCF at high density and temperature.

- GEANT4 studies of spallation neutron yield for ADS targets, Proc. IPAC 2016, TUPOY019.